Code Explanation – Smart Energy & Environmental Management System

This project is based on an ESP32 and is designed to monitor environmental conditions and automatically control devices. I am using a DHT22 sensor to measure temperature and humidity, an LDR to detect the light level, and a PIR sensor to detect whether a person is present.

Based on these sensor readings, the ESP32 controls a light, fan, LEDs, and a buzzer. An OLED display is also used to show the current status of the system.

Including the Required Libraries

#include <Wire.h>

#include <Adafruit_GFX.h>

#include <Adafruit_SSD1306.h>

#include "DHT.h"

First, I include the libraries required for the project.

Wire.h is used for I2C communication, which is required for the OLED display.

Adafruit_GFX.h provides the graphics and text functions for the display.

Adafruit_SSD1306.h is used to control the SSD1306 OLED display.

DHT.h is used to communicate with the DHT22 sensor and read temperature and humidity.

Sensor Pin Definitions

#define DHTPIN 4

#define DHTTYPE DHT22

#define PIR_PIN 27

#define LDR_PIN 34

Here, I define the GPIO pins connected to the sensors.

The DHT22 is connected to GPIO 4, the PIR sensor is connected to GPIO 27, and the LDR is connected to GPIO 34.

I also specify that the DHT sensor being used is a DHT22.

Output Pin Definitions

#define YELLOW_LED 18

#define BLUE_LED 19

#define RED_LED 23

#define BUZZER 5

#define LIGHT_RELAY 26

#define FAN_RELAY 25

These are the GPIO pins connected to the output devices.

The yellow LED represents the light status, the blue LED represents the fan status, and the red LED represents the high-temperature warning.

The buzzer provides an audible warning.

The light relay and fan relay are used to control the light and fan.

OLED Display Configuration

#define SCREEN_WIDTH 128

#define SCREEN_HEIGHT 64

The OLED display has a width of 128 pixels and a height of 64 pixels.

Then I create the OLED display object:

Adafruit_SSD1306 display(

SCREEN_WIDTH,

SCREEN_HEIGHT,

&Wire,

-1

);

This sets up the OLED using the I2C communication interface.

The -1 means that a separate reset pin is not being used.

Creating the DHT22 Object

DHT dht(DHTPIN, DHTTYPE);

Here, I create a DHT sensor object.

It uses GPIO 4 as the data pin and DHT22 as the sensor type.

This object is later used to read temperature and humidity.

Defining the Thresholds

#define DARK_THRESHOLD 1800

#define FAN_TEMPERATURE 30.0

#define WARNING_TEMPERATURE 38.0

These values are used by the decision-making part of the program.

The LDR reading is compared with DARK_THRESHOLD.

If the light level is below 1800, the program considers the room to be dark.

The fan temperature is set to 30°C. At or above this temperature, the fan turns ON.

The warning temperature is set to 38°C. At or above this temperature, the warning LED and buzzer turn ON.

The setup() Function

void setup(){

The setup() function runs once when the ESP32 starts.

Starting Serial Communication

Serial.begin(115200);

This starts communication with the Serial Monitor at a speed of 115200 baud.

I use the Serial Monitor to see the sensor readings and the current status of the system.

Starting the DHT22 Sensor

dht.begin();

This initializes the DHT22 sensor so that the ESP32 can start reading temperature and humidity.

Setting the Pin Modes

pinMode(LDR_PIN, INPUT);

pinMode(PIR_PIN, INPUT);

pinMode(YELLOW_LED, OUTPUT);

pinMode(BLUE_LED, OUTPUT);

pinMode(RED_LED, OUTPUT);

pinMode(BUZZER, OUTPUT);

pinMode(LIGHT_RELAY, OUTPUT);

pinMode(FAN_RELAY, OUTPUT);

Here I define which pins are inputs and which are outputs.

The LDR and PIR provide information to the ESP32, so they are configured as inputs.

The LEDs, buzzer, and relays are controlled by the ESP32, so they are configured as outputs.

Turning Everything OFF Initially

digitalWrite(LIGHT_RELAY, LOW);

digitalWrite(FAN_RELAY, LOW);

digitalWrite(YELLOW_LED, LOW);

digitalWrite(RED_LED, LOW);

digitalWrite(BLUE_LED, LOW);

digitalWrite(BUZZER, LOW);

At startup, I keep all the output devices OFF.

This gives the system a known starting condition and prevents the devices from being activated unexpectedly when the ESP32 is powered on.

Initializing the OLED Display

if (!display.begin(SSD1306_SWITCHCAPVCC, 0x3C)) {

Serial.println("OLED initialization failed!");

while (true);

}

Here, the program tries to initialize the OLED display.

The address 0x3C is the I2C address being used for the OLED.

If the display cannot be initialized, the program prints an error message:

OLED initialization failed!

Then:

while (true);

keeps the program stopped because the display is not working.

Displaying the Startup Message

display.clearDisplay();

display.setTextSize(1);

display.setTextColor(SSD1306_WHITE);

display.setCursor(0, 0);

display.println("Smart Energy System");

display.setCursor(0, 20);

display.println("Starting...");

display.display();

delay(2000);

After successfully initializing the OLED, I clear the screen and configure the text.

The OLED first displays:

Smart Energy System

Starting...

The display.display() command actually sends the prepared content to the OLED.

The program then waits for two seconds.

Starting Message in the Serial Monitor

Serial.println("------------------------------");

Serial.println("SMART ENERGY MANAGEMENT SYSTEM");

Serial.println("------------------------------");

This prints a heading in the Serial Monitor so that I can easily identify the output from this project.

The loop() Function

void loop(){

The loop() function is the main part of the program.

Unlike setup(), this function keeps running repeatedly.

The basic sequence is:

Read sensors → Check conditions → Control devices → Show results → Wait → Repeat

Reading the DHT22

float temperature = dht.readTemperature();

float humidity = dht.readHumidity();

The DHT22 provides two readings.

temperature stores the current temperature in Celsius.

humidity stores the current relative humidity percentage.

Reading the LDR

int lightLevel = analogRead(LDR_PIN);

The LDR is an analog sensor, so I use analogRead() to obtain its current value.

The value is stored in lightLevel.

This value is later compared with the dark threshold to determine whether the room is dark or bright.

Reading the PIR Sensor

int motion = digitalRead(PIR_PIN);

bool personDetected = (motion == HIGH);

The PIR sensor gives a digital signal.

If the signal is HIGH, the program considers that a person has been detected.

The Boolean variable personDetected therefore becomes either true or false.

Checking Whether the DHT22 Reading Is Valid

if (isnan(temperature) || isnan(humidity)) {

Serial.println("DHT22 reading failed!");

delay(2000);

return;

}

Before continuing, I check whether the DHT22 returned valid values.

isnan() checks whether a value is not a valid number.

If either temperature or humidity fails:

- An error is printed.

- The program waits for two seconds.

- return stops the current loop iteration.

The ESP32 will then try reading the sensor again during the next loop.

Determining Whether the Room Is Dark

bool roomDark = (lightLevel < DARK_THRESHOLD);

bool lightON = false;

Here, the LDR value is compared with the dark threshold.

If:

lightLevel < 1800

then roomDark becomes true.

Otherwise, it becomes false.

I initially set lightON to false so that the light remains OFF unless the required conditions are satisfied.

Automatic Light Decision

if(roomDark && personDetected){

lightON=true;

}

else{

lightON=false;

}

This is an important part of the project.

The && means AND.

Therefore, both conditions must be true:

- The room must be dark.

- A person must be detected.

Only then does the light turn ON.

So the logic is:

Dark + Person detected → Light ON

Dark + No person       → Light OFF

Bright + Person        → Light OFF

Bright + No person     → Light OFF

Controlling the Light

digitalWrite(YELLOW_LED,lightON);

digitalWrite(LIGHT_RELAY,lightON);

The result of the previous decision is sent to both the yellow LED and the light relay.

If lightON is true, both are turned ON.

If it is false, both are turned OFF.

The yellow LED therefore acts as an indicator of the light status.

Fan Control

bool fanON=false;

I initially keep the fan OFF.

Then the temperature is checked:

if(temperature >= FAN_TEMPERATURE){

fanON=true;

}

else{

fanON=false;

}

If the temperature is 30°C or higher, fanON becomes true.

Otherwise, it remains false.

Then the fan output is controlled:

digitalWrite(BLUE_LED,fanON);

digitalWrite(FAN_RELAY,fanON);

The blue LED indicates the fan status.

The fan logic is therefore:

Temperature < 30°C → Fan OFF

Temperature ≥ 30°C → Fan ON

High-Temperature Warning

bool warningON=false;

The warning is initially OFF.

Then the program checks the warning threshold:

if(temperature >= WARNING_TEMPERATURE){

warningON=true;

}

else{

warningON=false;

}

If the temperature reaches or exceeds 38°C, the warning becomes active.

Red LED Warning

digitalWrite(RED_LED,warningON);

The red LED follows the warning condition.

Therefore:

Temperature < 38°C → Red LED OFF

Temperature ≥ 38°C → Red LED ON

This provides a visual warning that the temperature is high.

Buzzer Warning

digitalWrite(BUZZER,warningON);

The buzzer uses exactly the same warning condition as the red LED.

So when the temperature reaches 38°C or above, both the red LED and buzzer are activated.

This gives the user both a visual and audible warning.

Printing the Sensor Values to the Serial Monitor

The next section prints the current readings and system status.

Serial.println();

Serial.println("------------------------------");

Serial.print("Temperature: ");

Serial.print(temperature);

Serial.println(" C");

This prints the current temperature.

Then humidity:

Serial.print("Humidity: ");

Serial.print(humidity);

Serial.println(" %");

Then the LDR value:

Serial.print("Light Level: ");

Serial.print(lightLevel);

This lets me see the raw light sensor reading while testing.

Showing Person Detection

Serial.print("Person: ");

if(personDetected){

Serial.println("YES");

}

else{

Serial.println("NO");

}

This prints whether the PIR sensor has detected a person.

For example:

Person: YES

or:

Person: NO

Showing the Room Condition

Serial.print("Room: ");

if(roomDark){

Serial.println("DARK");

}

else{

Serial.println("BRIGHT");

}

This converts the LDR reading into a simple description.

Instead of only showing a numerical LDR value, the Serial Monitor tells me whether the program considers the room dark or bright.

Showing Light Status

Serial.print("Light: ");

if(lightON){

Serial.println("ON");

}

else{

Serial.println("OFF");

}

This displays whether the automatic light-control condition is currently active.

Showing Fan Status

Serial.print("Fan: ");

if(fanON){

Serial.println("ON");

}

else{

Serial.println("OFF");

}

This shows whether the temperature condition has caused the fan to turn ON.

Showing Warning Status

Serial.print("Warning: ");

if(warningON){

Serial.println("ON");

}

else{

Serial.println("OFF");

}

This displays whether the high-temperature warning is currently active.

So the Serial Monitor gives me a complete view of what the system is doing.

Updating the OLED Display

The next section displays the important information on the OLED.

First, the previous screen is cleared:

display.clearDisplay();

Then the text size and color are configured:

display.setTextSize(1);

display.setTextColor(SSD1306_WHITE);

The system title is displayed:

display.setCursor(0, 0);

display.println("SMART ENERGY SYSTEM");

Displaying Temperature

display.setCursor(0, 12);

display.print("Temp: ");

display.print(temperature, 1);

display.println(" C");

The current temperature is shown on the OLED.

The 1 in:

temperature, 1

means that the temperature is displayed with one decimal place.

For example:

Temp: 31.5 C

Displaying Humidity

display.setCursor(0, 22);

display.print("Humidity: ");

display.print(humidity, 1);

display.println("%");

The current humidity is displayed with one decimal place.

For example:

Humidity: 62.4%

Displaying Light Status

display.setCursor(0, 32);

display.print("Light: ");

if (lightON) {

display.println("ON");

} else {

display.println("OFF");

}

The OLED shows whether the automatic light system is currently ON or OFF.

Displaying Fan Status

display.setCursor(0, 42);

display.print("Fan: ");

if (fanON) {

display.println("ON");

} else {

display.println("OFF");

}

This displays the current fan status.

For example:

Fan: ON

when the temperature is at or above 30°C.

Displaying the Final Status Message

The last line of the OLED changes depending on the current situation:

display.setCursor(0, 52);

if (warningON) {

display.println("WARNING!");

} else if (personDetected) {

display.println("Person detected");

} else {

display.println("No person");

}

There are three possible messages.

If the temperature is at least 38°C:

WARNING!

is displayed.

If there is no warning but the PIR detects a person:

Person detected

is displayed.

Otherwise:

No person

is displayed.

The warning condition is checked first, so the warning message gets priority over the person-detection message.

Sending the Information to the OLED

display.display();

All the text prepared above is finally sent to the OLED screen using display.display().

Without this command, the information would not actually appear on the display.

Two-Second Delay

At the end:

delay(2000);

The ESP32 waits for two seconds.

After that, the loop() starts again.

So the system continuously repeats this process:

Read → Decide → Control → Display → Wait → Repeat

Overall Logic of the Project

The complete decision-making process is:

DHT22

Temperature/Humidity

│

↓

ESP32

│

┌─────────┴─────────┐

│                                                     │

Temperature                                  LDR + PIR

│                                                     │

↓                                                       ↓

≥ 30°C?                                   Dark + Person?

│                                                   │

YES                                            YES

│                                                  │

↓                                                   ↓

Fan ON                                     Light ON

Blue LED ON                        Yellow LED ON

Fan Relay ON                       Light Relay ON

│

│

Temperature ≥ 38°C?

│

YES

│

↓

Red LED ON + Buzzer ON

The OLED and Serial Monitor continuously show the current state of the system.

Simple Summary of the Main Conditions

| Sensor Condition Room is dark + person detected → Light ON Room is bright → Light OFF No person detected → Light OFF Temperature ≥ 30°C → Fan ON Temperature < 30°C → Fan OFF Temperature ≥ 38°C → Red LED + Buzzer ON Temperature < 38°C → Warning OFF |  |
| --- | --- |

Final Conclusion

In this program, the ESP32 works as the main controller. It continuously receives information from the DHT22, LDR, and PIR sensors and uses that information to make automatic decisions.

The light is controlled based on both the room's brightness and the presence of a person. The fan is controlled according to temperature, and a separate warning system is activated when the temperature becomes very high.

The OLED display provides the user with important information, while the Serial Monitor helps monitor the sensor readings and system status during testing.

Overall, this code demonstrates how different sensors and output devices can be combined with an ESP32 to create a simple automated energy and environmental management system.